三菱伺服说明书 MR-J2-B伺服放大器手册(英文)

8 - 1 Alarm and warning lists    报警和警告名单

When a fault occurs during operation, the corresponding alarm or warning is displayed. If any alarm or

warning has occurred, refer to Section 8.2 or 8.3 and take the appropriate action.Alarms Warnings：当故障发生在操作过程中，相应的报警或显示警告。如果任何警报或警告发生，请参阅第8.2或8.3，并采取适当的行动。报警警告

Display Name 显示名称

10 Undervoltage                           10欠压

11 Board error 1                            11 局错误1

12 Memory error                           12内存错误1

13 Clock error                              14时钟误差

15 Memory error 2                           15 内存错误2

16 Encoder error 1                           16 编码器错误1

17 Board error 2                             17局错误2

18 Board error 3                             18局的错误3

20 Encoder error 2                           20编码器错误2

24 Ground fault                             24接地故障

25 Absolute position erase                     25绝对位置擦除

30 Regenerative error                         3 0再生错误

31 Overspeed                                31超速

32 Overcurrent                               32过流

33 Overvoltage                               33过压保护

34 CRC error                                 34 CRC错误

35 Command F T error                          35指挥F t误差

36 Transfer error                               36传输错误

37 Parameter error                              37参数错误

46 Servo motor overheat                         46伺服电机过热

50 Overload 1                                  50超载1

51 Overload 2                                 51超载2

52 Error excessive                             52错误过多

8E RS-232C error                              8E型的RS - 232错误

88 Watchdog                                 88看门狗

92 Open battery cable warning                   92打开电池电缆警告

96 Zero setting error                           96零设定错误过度负荷的

E0 Excessive regenerative load warning             E0再生警告

E1 Overload warning                             E1超载警告

E3 Absolute position counter warning             E3展绝对位置计数器警告

E4 Parameter warning                            E4类参数警告

E6 Servo emergency stop                       E6伺服紧急停止

E7 Controller emergency stop                    E7的紧急停止控制器

E9 Main circuit off warning                      E9主回路关闭警告